ClassicGlasTM

Fiberglass Columns

Split and Tabbed for Reassembly Installation Instructions

Installer Supplied Materials:

1. Non-corrosive wood screws.

2. Premium grade polyurethane adhesive.

3. Fiberglass repair materials similar to marine grade bondo.

4. Metal "L" brackets and non corrosive nuts and bolts.

Overview:

Columns will arrive with shafts, capitals and bases split in halves. One shaft half will have a full length alignment and attachment tab on both seams. Shaft halves will be screwed together when they arrive. Capitals and bases will be provided with alignment tabs. Split columns are non-load bearing.

Reassembly Instructions:

1. Prior to erecting halves, pre-fit them on the ground. Column halves may not retain semicircular shape when disassembled.  If necessary, a plywood circle the same size as the column shaft can be attached to the interior of the column shaft, or structural support with blocking and glued inside shaft during installation to aid with realignment of column halves.

2. Pre-drill holes for attachment of the “L” brackets to the top and bottom of both of the column shaft halves. Attach “L” brackets to the top and bottom of both shaft halves using through bolts by others. Do not screw into the column shaft, use through bolts.

3. Secure the column shaft half with the tabs in place using non-corrosive fasteners and metal "L" brackets at top and bottom of shaft. Do not place seam to front.

4. Lift the second column shaft half into place approximately 3 inches from the first shaft half.

5. Apply high quality polyurethane adhesive to the shaft alignment tab and seam of both shaft halves. Quickly align shaft halves around the interior support, and join them together using the small screws included. Wipe off excess adhesive from the surface of the column immediately. Using the "L" brackets on the second shaft half to the structure using non-corrosive fasteners.

7. Follow adhesive manufacturers curing guidelines. Be sure to account for cool or damp conditions. Upon completion of curing remove screws, fill screw holes and feather out shaft joint seam using a fiberglass repair kit (Bondo). After curing, sand with 120 grit, or finer sandpaper.

8. Install capital and base/plinth halves in place using non-corrosive fasteners, blocking and adhesive.

9. After installation of capital and base, repair any nicks, screw holes or seams with fiberglass repair kit (Bondo) and sand smooth with 120 grit or finer sandpaper.

10. Insert polypropylene backer rope into joint between column capital and base where capital and base meet the column shaft. Caulk joint where capital and base meet shaft with premium grade paintable caulk.

11. Clean column shaft thoroughly to remove and oil, grease, or dirt prior to painting.

12. Immediately after installation, apply high quality latex primer and a minimum of two coats of Duration Acrylic paint to all exterior surfaces.